WEEK 3 TUTORIAL – PROBLEM SOLVING PROCESS

Step 1: Understand and Define the Problem (Analyse)

The goal is to design a combinational logic circuit to activate a LOW level alarm signal whenever:

- Car ignition is ON, and

- Either driver or passenger seat is occupied, and

- Either seatbelt of occupied seats is not fastened.

Step 2: Organise and Describe the Data

Input Data:

- DRIV = driver in seat

- PAS = passenger in seat

- BELTD = driver’s seatbelt is unfastened

- BELTP = passenger’s seatbelt is unfastened

- IGN = Ignition switch on

Output Data:

- Alarm = active LOW when sound is 0

Step 3: Plan the Solution (Design the Algorithm)

3.1 Algorithm (in plain English)

INPUT:

DRIV  = driver seat

PAS = passenger seat

IGN = ignition

BELTD = driver seatbelt

BELTP = passenger seatbelt

PROCESS:

IF DRIV and PAS are occupied
AND BELTD and BELTP are fastened
AND IGN is turned on
THEN ALARM will not trigger

IF ELSE
THEN ALARM will trigger LOW

3.2 Truth Table

| IGN | DRIV | ^BELTD | PAS | ^BELTP | ALARM |
| --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0 | 0 | 1 |
| 1 | 0 | 0 | 0 | 1 | 1 |
| 1 | 0 | 0 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 | 1 | 1 |
| 1 | 0 | 1 | 0 | 0 | 1 |
| 1 | 0 | 1 | 0 | 1 | 1 |
| 1 | 0 | 1 | 1 | 0 | 1 |
| 1 | 0 | 1 | 1 | 1 | 1 |
| 1 | 1 | 0 | 0 | 0 | 0 |
| 1 | 1 | 0 | 0 | 1 | 0 |
| 1 | 1 | 0 | 1 | 0 | 0 |
| 1 | 1 | 0 | 1 | 1 | 0 |
| 1 | 1 | 1 | 0 | 0 | 1 |
| 1 | 1 | 1 | 0 | 1 | 1 |
| 1 | 1 | 1 | 1 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 |

GREEN = will not trigger

IGN is always on

3.3 Boolean Expressions (SOP)

> DRIV. BELTD. PAS. BELTP.IGN

> DRIV.^BELTD.^PAS.BELTP.IGN

> DRIV.^BELTD.PAS.^BELTP.IGN

> DRIV.^BELTD.PAS.BELTP.IGN

> DRIV.BELTD.PAS.^BELTP.IGN

ALARM = DRIV. BELTD. PAS. BELTP.IGN + DRIV.^BELTD.^PAS.BELTP.IGN + DRIV.^BELTD.PAS.^BELTP.IGN + DRIV.^BELTD.PAS.BELTP.IGN + DRIV.BELTD.PAS.^BELTP.IGN

3.4 Flowchart

3.5 Pseudocode

START

GET IGN,DRIV,BELTD,PAS,BELTP

IF DRIV. BELTD. PAS. BELTP.IGN + DRIV.^BELTD.^PAS.BELTP.IGN + DRIV.^BELTD.PAS.^BELTP.IGN + DRIV.^BELTD.PAS.BELTP.IGN + DRIV.BELTD.PAS.^BELTP.IGN

THEN ALARM = 0 #ON

ELSE ALARM = 1 #OFF

END

Step 4: Test and Refine the Solution (Debug and Verify)

| IGN | DRIV | ^BELTD | PAS | ^BELTP | ALARM | OUTPUT |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0 | 0 | 1 |  |
| 1 | 0 | 0 | 0 | 1 | 1 |  |
| 1 | 0 | 0 | 1 | 0 | 1 |  |
| 1 | 0 | 0 | 1 | 1 | 1 |  |
| 1 | 0 | 1 | 0 | 0 | 1 |  |
| 1 | 0 | 1 | 0 | 1 | 1 |  |
| 1 | 0 | 1 | 1 | 0 | 1 |  |
| 1 | 0 | 1 | 1 | 1 | 1 |  |
| 1 | 1 | 0 | 0 | 0 | 0 |  |
| 1 | 1 | 0 | 0 | 1 | 0 |  |
| 1 | 1 | 0 | 1 | 0 | 0 |  |
| 1 | 1 | 0 | 1 | 1 | 0 |  |
| 1 | 1 | 1 | 0 | 0 | 1 |  |
| 1 | 1 | 1 | 0 | 1 | 1 |  |
| 1 | 1 | 1 | 1 | 0 | 0 |  |
| 1 | 1 | 1 | 1 | 1 | 1 |  |

Part 3: Implementing the Solution